Table 9.4    Endowment of Natural Resources: Block wise Information

| Sl. No | # Particulars | ## Kumbakonam | ## Vikkravandi | ## Papireddipatti | ## Bhavani | ## Coonoor | ## Thiruppulani | ## Killiyur |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | River & Drainage | Yes | Yes | Yes | Yes | Yes | No | Yes |
| 2 | Rainfall (Max) | 1881 | 1305 | 1640.6 | 922.6 | 2189 | 1185 | 1329 |
|  | (Min) | 668 | 204 | 636.4 | 424 | 1118 | 128 | 689 |
| 3 | Geology | Levee Sand, | Hornblende | Hard Cryslalline Rock | Hard and | Cryslalline Rock | Alluvium | Garnet, Sillimanate |
|  |  | and Clay | Biotite, Gneiss | Charnokite | Flood Plains | of Archean age | Terrain | Graphite Gneiss |
|  |  |  | Charnokite | Foliated Gnesses | of Archean age | and full of | Sedements | and Garnet |
|  |  |  | Alluvium |  | Gneiss Granites | Charnokite |  | Biotite gneiss |
|  |  |  |  |  | and Pyroxenite |  |  |  |
| 4 | Geomorphology | Flood plains | Pediments | Burial Pediments Deep | Donudational | Deflection Sloppe, | Deltaic Plains | Less Dissected |
|  |  | Delta Plains | Burial Pediments | Burial Pediments | hills and Deep | Less | Sand Dunes | Plateaus.Sedimenary |
|  |  | Levee complexes | Deep and | Sallow Valley Hill and | Buried Pediments | Dissected Plateau | Beach Ridges | Plains and |
|  |  |  | Flood Plains | Hill top Sediments | and Flood Plains |  |  | Coastal Plains |
| 5 | Water level (Summer) | 4.7 - 5.8 | 10.3 - 16.6 | 2.94 - 9.47 | 6.27 - 11.5 | 1.47 - 2.68 | 2.75 - 8.56 | 19.08 - 35.20 |
|  | Meter (Winter) | 1.6 - 2.6 | 7.03 - 15.65 | 4.41 - 19.07 | 1.61 - 8.07 | 2.00 - 2.91 | 2.91 - 4.15 | 19.06 - 35.45 |
| 6 | Water Quality Ec-micromhos/cm | 740 - 1340 | 1500 - 4000 | 925 - 1375 | 1626 - 1762 | 170 - 660 | 540 - 13000 | 500 - 1000 |
| 7 | Soil Group (A) | 87 | 16 | 33 | 18 | 0 | 29.91 | 0 |
|  | Soil Group (B) | 12 | 50 | 33 | 75 | 85.88 | 17.62 | 52 |
|  | Soil Group (C) | 1 | 34 | 34 | 70 | 14.12 | 18.75 | 48 |
|  | Soil Group (D) | 0 | 0 | 0 | 0 | 0 | 33.72 | 0 |
| 8 | Slope | Level Slope | Level Slope | Steep Slope | Very Gently | Terrain Mostly | Flat Terrain with | Moderately |
|  |  |  |  | Strongly Slope | Sloping | Sloping | Gentle Slope | Sloping |
| 9 | Land Use (Plant) | 69 | 34 | 35 | 68 | 67 | 77.25 | 95 |
|  | (Settlement) | 22 | 6 | 15 | 8 | 7 | 0.77 | 4 |
|  | (Water Bodies) | 7 | 59 | 50 | 8 | 1 | 9.43 | 1 |
|  | (Waste Lands) | 2 | 1 | 0 | 16 | 25 | 12.55 | 0 |

Source: Institute of Remote Sensing, Anna University, Chennai, 1999