## WATER

Rain Water Harvesting

###### MIZORAM

### Dunglena

Mizoram, one of the smallest states of India has an area of 21000 sq kms.  The state is entirely mountainous covered with green vegetation. The mountain ranges in North-South directions and as such the rivers flow either in North or South directions. Towns and villages are mostly located on hilltops or on the upper reaches of the hills.  Perennial streams and rivers being much below habitations, scarcity of water in dry season is very common.

Traditional Water Supply

Springs on the hills slopes and valleys used to be main sources of drinking water.  During season most of the springs dried up or the yield got reduced considerably.  People walk long distances to fetch a bucket of water in dry season.  In the year 1900 the British who established an outpost at Aizawl constructed a ground reservoir of 12 lakh gallons capacity on a hilltop.  A sloped roof of corrugated galvanized iron sheets on timber frames was constructed around the reservoir for rainwater catchment.  This was the only water supply of Aizawl for government officials till the year 1973.  From this time the Mizo people practiced rooftop rainwater harvesting to meet their domestic water demands.  Spring water supplemented by rainwater harvesting is still the means of water supply in many villages and towns.  Many people in Aizawl, the state capital, also depend on roof top rainwater collected on to meet their domestic demands.

##### Rain Water

Mizoram has an average rainfall of about 250 cms per annum.  The rainy season of six months in a year is between May and October.  The pattern of rainfall recorded in Aizawl is shown as follows:

| Year | 1988 | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rainfall (cm) | 265.6 | 282.7 | 261.4 | 194.5 | 322.8 | 207 | 250.1 | 236.3 |

Rainwater in Mizoram is free from undesirable impurities.   If properly stored it remains pure chemically.  However, mosquito breeding and bacterial growth develop on long storage requiring disinfections.

##### Types of Rainwater Harvesting

Houses in Mizoram are constructed with slope roof.  Even in Villages, thatch roofs are replaced by galvanized Corrugated Iron (GCI) Sheets.  Semi circular rain gutter made with plain galvanized sheets tilted on the caves of roof collect rainfall and stored in reservoirs for use in the dry season.  Circular GP sheets are very common.  Masonry ground tanks and reinforced cement concrete tanks of varying sizes are also used.  Scarcity of good sand and stone in Mizoram restricted use of concrete and ferro-cement tanks

International Drinking Water Decade Programme (1981-1990)

During this decade programme a massive programmes of providing drinking water to all problem villages was launched.  As most of the towns and villages in Mizoram are on high hill slopes or hill tops gravity water sources and ground water sources are very scanty.  Government of Mizoram Public Health Engineering Department proposed springs improvements and rooftop water harvesting to solve the drinking water supply problems of Mizoram. The Government of India, under the National Drinking Water Mission approved the schemes in the Accelerated Rural Water Supply Programme (ARWSP) and State Plan of Mizoram. From the year 1986 rainwater Harvesting and Spring Improvement Schemes were taken up each year. So far the Government of Mizoram provided about 15000 nos. of rainwater harvesting tanks to individual households in over 200 villages of Mizoram. The number of rainwater harvesting tanks constructed by private individuals in the different places of Mizoram may be around 50000 tanks.

Initially common public water reservoirs by harvesting rain from the roofs of town halls and churches were constructed. Distribution and control of water by villagers themselves was very difficult. Thereafter, individual house tanks were designed and found to be workable and were welcomed by the people as well.

Design Parameters

Rainwater Harvesting depends on two things

- Quality of Rainfall

- Area and nature of catchment

The catchment may be classified into three categories

- Hard surfaces like roofs and rocks which gives a total run off-100%

- Semi hard surfaces like pavements compounds of houses with 50% run off

- Loose soil which gives 25% off

The following simple formula may be applied to arrive at the quantity of rainwater collection:

Quantity of water = Catchment Area x run–off factor x rainfall

For example:

- If the roof area is 10 cm and 5cm and rainfall is 250 cm per annum:

Quantity of rainfall = 10 x 5 x 250/100 x 1000 = 125,000 litres per annum

- For a family of 8 persons, this is 125000/8 x 365 = 45 litres per capita per day

Individual rainwater harvesting tanks provided by the Government of Mizoram under the Rural Water Supply Programmes are designed assuming a family of eight persons per day at a drinking water supply rate of 10 litres per capita per day and storage required for 120 days of continuous non railway days in a year

Size of tank = 8 x 10 x 120 = 9600 litres

Provide: 10,000 litres capacity GP Circular tank

Quality Control

Atmosphere of Mizoram particularly in rural areas is free from air pollution.  Rainwater is free from chemical impurities.  The first rainfall for an hour, after dry season, should not be collected to avoid dirt from roof entering the water tank.  Boiling of water for direct consumption is the normal practice.  Mosquito breeding after long storage is very common and some bacterial growth can be expected.  An occasional dose of bleaching powder once in two months is recommended.  Mizo people keep rainwater in the storage tanks for months and even few years and the rainwater collected remains good for long period of time.

Conclusion

The process of evaporation and precipitation is a continuous process going on all the time. As such, rainwater is renewable.  Most of the rainfall is being wasted.  If properly harvested, the rainfall received is sufficient to meet our demands.  If everyone at every place practices rainwater harvesting the crisis of water can be solved.  The crisis is not about having too little water to satisfy our needs but a crisis of bad management of water resources.