CHAPTER - 1

INTRODUCTION

1.1	Origin of the System

The Upper Ganga Canal (UGC) system commissioned as far back as 1854-55 has its origin from the mythological Ganga.  The Ganga rises in the Gangotri glacier in the Himalayas at an altitude of 7010 M above mean sea level in the Uttarkashi district of Uttraranchal. The river is called Bhagirathi at its source.  Descending down the valley it is joined by the Alaknanda at Dev Prayag; the Bhagirathi Kharak and Satopanth.  Below confluence with Alakananda, river is called the Ganga.  Cascading approximately 160 km and cutting through the Shivalik range of hills it emerges into the plains at Haridwar.

The Ganga basin extends over an area of 107,331 sq.km and area in erstwhile Uttar Pradesh was 71302 sq.km.  The catchment area upto Bhimgoda site from where the Upper Ganga Canal takes off is 25.28 lakh hectares.

Annual run off at Raiwala (75% availability) is minimum 169 cumecs & maximum 2086 cumecs.

1.2	Historical Background

Upper Ganga Canal - The construction of Upper Ganga Canal was conceived & constructed by Proby T.  Cautley during the period 1840-1854.  In the beginning one of the branches of river - a natural channel flowing near Haridwar - was made use of to divert practically the entire winter flow by construction of temporary obstructions across other branches.

Mr. Proby.T.Cautley designed and constructed a versatile network incorporating almost all types of engineering cross-drainage marvels & regulating structures.  He has been affectionately remembered as a British Engineer with an Indian heart.  This arrangement continued to work for almost fifty long years.  With increase in demand, the state took up construction of a permanent headworks in 1913 and completed it in 1920.  It consisted of a weir about 550 m long fitted with 1.8 m high falling shutters & located about 3 km upstream of old regulator.  The UGC system then comprised 910 km of main canal and branches and 5280 km of distributaries to provide irrigation facilities in the district of Saharanpur, Muzaffarnagar, Meerut, Bulandshaher & Aligarh; total area irrigated annually being 0.7 million hectares.  The index map showing districts in UGC command is given at  Dwg. 1.1.  Photographs taken at selected sites are shown in Fig. 1 to Fig. 9.

The Upper Ganga Canal takes off from the right flank of Bhimgoda barrage which replaced the old weir at Haridwar in 1991-92. The canal with a head discharge of 190 cumecs (6750 cusecs) presently provides irrigation in a gross command area of about 20 lakh ha. in 10 districts of Western Uttar Pradesh. There are 4 major cross drainage works in initial 36 kms of the main canal. In the revised proposal, the canal has to carry an increased discharge of 295 cumecs (10419 cusecs). The maximum capacity of the canal in head reaches is proposed to be as 370 cumecs (13068 cusecs) which includes 20% extra inflow for silt ejector. The 4 cross drainage works located on the old canal are more than a century old and have outlived their lives. As such the Upper Ganga Irrigation Modernisation Project proposes to modernize the Upper Ganga Canal System in 4 phases in a time span of 40 years.

The first time slice of phase- I proposes to construct a parallel lined canal (Parallel Upper Ganga Canal) of 295 cumecs capacity beyond silt ejector from 6 to 36 km and 177.5 to 240 km, 4 new cross drainage works on the Parallel Upper Ganga Canal (PUGC) namely Ranipur Syphon, Pathri Super Passage, Ratmau Aqueduct and Solani Aqueduct ; modernization of 3 distributaries (Dy.) namely Bulandshahar Dy., Harduaganj Dy. & Machhua Dy. in the districts of Bulandshahar and Aligarh and other related works. The project will also provide an additional potential of 9000 ha in the districts of Aligarh and Bulandshahar.

  - Madhya Ganga Irrigation project

The Madhya Ganga Canal project was formulated for utilization of surplus monsoon waters of the Ganga during Kharif for development of paddy cultivation in the two dry pockets between Anupshahr branch & Upper Ganga Canal and between Upper Ganga Canal & Karwan Nadi in the lower command of Upper Ganga Canal. The project envisages construction of a barrage across the Ganga near Bijnour, Main Madhya Ganga Canal off- taking from the right bank and terminating in the Upper Ganga Canal, Lakhaoti branch with its distribution system, Mat branch feeder with its distribution system, Amarpur Dy. with its distribution system, Parallel Mat branch and Parallel Hathras branch Canal and remodeling of existing Mat, Hathras and Anupshahr branch canals of Upper Ganga Canal System.

The project will provide annual irrigation to 178 thousand  ha. of which 64 thousand ha. will be in the new command areas of Ghaziabad, Bulandshahar & Aligarh districts and balance in the existing command of Upper Ganga Canal in the districts of Saharanpur, Muzaffarnagar, Meerut, Ghaziabad, Bulandshahar, Aligarh, Mathura, Agra, Etah & Mainpuri in the western part of Ganga-Yamuna Doab.. The project was taken up for execution in 1977 and was targeted for completion by December 2000. The revised schedule of completion is by 2002-03.

Upper Ganga Canal is comprised of about 7000 k.m. of canal network of branches, distributaries & minors to deliver water for irrigation, canal -based drinking water supply and for other miscellaneous usage. (Dwg.1.2)

1.4	UPPER GANGA CANAL SYSTEM

The main branches are : -

- Deoband branch

- Anupshahar branch

- Mat branch

- Hathras branch; taking off from Mat branch.

U.G.C. in its tail portion or a channel taking off near tail of UGC is called supply channel & has existing capacity of 15.49 cumecs.

Unlike Western Jamuna Canal (W.J.C.) where the distribution network has been designed and constructed for a very large C.C.A. both on the main W.J.C. command & projects like JLN, Loharu & Jui Lift Irrigation Systems and supplies available during lean months of winter are a fraction of total requirement necessitating four major groups & internal grouping to receive supply during first preference turn; the Upper Ganga Canal System has a comparatively better availability of supplies.  Against capacity of around 311.52 cumecs at head in Parallel Upper Ganga Canal (PUGC) proposed to be supplemented with discharge of over 56.64 cumecs from augmentation tubewells (ATW) & allowing for losses supplies available even during lean months are more than half the total requirement.  The full supply capacity of the system including designed discharge of ATWs would work out to around 368.16 cumecs whereas capacity of offtaking channels is 437.96 cumecs  which is likely to enhance gradually with system improvement & modernisation.  Even with availability of full supply discharge, pro-rata discharge for distribution thus works out to about 84%.  The capacity of offtake & distribution network exceeds 453.12 cumecs whereas capacity of conveyance system including augmentation tubewells is limited to around 368.16 cumecs.  This is under conditions of better availability as such rotational running is necessary. But existing scenario of wanting maintenance, lack of control on unauthorised acts can be replaced with  better water management & strict enforcement for resetting of canal system in order.

1.5	New Okhla Barrage

Salient Features

| Pond Level | 201.35 M |
| --- | --- |
| Width of River | 445.73 M |
| Length of Barrage | 552 M |
| No. of Under Sluice Bays | 5 |
| No. of Barrage Bays | 22 |
| Width of under sluice bays | 18.3 M each |
| Width of barrage bays | 18.3 M each |
| Thickness of intermediate piers | 2.13 M |

Gates

a.	Number			27

b.	Size for under sluices	5 of 18.3 x 6 m each

for other			22 of 18.3 x 5.1 m each

Sediment excluder device		high excluder

Length of U/s divide wall		85.42 M

Ruggry dissipataction array	Baffle blocks & dentated sills

Canal Head Regulator

Width of head regulator	80.57 M

Width of  bays		7.65 M each

Thickness of piers		1.5 M each

Orientation w.e.f.  barrage Axis			100

Location of head regulator from barrage		22.25 m W/s

Right bank

Max. discharge of canal - 242.4 cumecs

1.6	ALLOCATION OF WATERS

1.6.1	Criteria for Distribution of Supplies

The sharing of water from river Ganga is done as per standing orders issued by C.E. incharge & it lays down the principles for inter-circle and inter-divisional distribution of water in U.G.C. Command.

1.6.2	Procedure for Distribution of Water

On the U.G.C. Command, Chief Engineer (Upper Ganga Canal) Meerut, convenes a meeting of all the superintending engineers before the start of Rabi season to assess & decide probable availability  of water for utilization in UGC & makes weekly allocations to each circle incharge who in turn decides the plan of operation of branches / distributaries & minors under his charge.

Computations for allotment of water during Kharif for each division are also done.  Typical procedure adopted is shown at Annex 1.4 and 1.5. Releases are however subject to changes depending upon actual river flows available on weekly basis and quantities released in excess or short are adjusted in the next week or subsequent week.

The main line conveyance losses including those in branches are also spread out canal division justification-wise for the periods I, II & III and the % age share of each division in the available waters of U.G.C. is decided: typical example are as below :

1.6.3	Share of Various Canal Division of U.G.C. During Kharif

Percentage share during Kharif        (Typical example)

| Name of Division | Period I | Period II | Period III |
| --- | --- | --- | --- |
|  | (1/4 to 19/5) | (20/5 to 17/6) | (18/6 to start of Monsoons |
| 1. Muzaffarnager Div. | 32.8 | 25.1 | 25.9 |
| 2. Anupshahar Branch Divn. Meerut | 16.8 | 14.8 | 14.6 |
| 3. Anupshahar Branch Narora Divn. | 1.7 | 3.9 | 3.6 |
| 4. Meerut Div. | 21.9 | 17.2 | 17.0 |
| 5. Bulandshahr Div. | 6.7 | 8.9 | 8.9 |
| 6. Mat Branch - Div. | 10.9 | 16.3 | 14.6 |
| 7. Aligarh Div. | 9.2 | 13.8 | 15.4 |

Typical assessment for Rabi have similarly been shown at Annex. 1.1 & 1.3.

Daily account of supplies received & due are done & this has been shown in Annex. 1.2.

In case of Rabi crop season; the mainline losses (31.15 Cumecs) & supplies delivered to Delhi Water Supply undertaking are deducted at source for the share of U.G.C. available at Dhanauri.

The conveyance losses in the branches & supply channels are however    incorporated in the shares of each division.

1.6.4	Share of Various Divisions of UGC during Rabi

(Typical example)

| S.No. | Name of Division | % of Share |
| --- | --- | --- |
| 1. | Muzaffar Nagar* | 16.0 |
| 2. | AB Divn. Meerut** | 12.0 |
| 3. | AB Narora | 5.40 |
| 4. | Meerut | 9.5 |
| 5. | Bulandshahar Division | 9.8 |
| 6. | Mat Branch Division | 30.0 |
| 7. | Aligarh Division *** | 17.3 |

*      Line Diagram of canal system (Dwg. 1.4) enclosed

**     Line Diagram of canal system (Dwg. 1.3) enclosed

***   Line Diagram of canal system (Dwg. 1.5) enclosed

The Divisional Officer / Executive Engineer prepares a roster of  running different distributaries & minors in his division as soon as he knows his anticipated share.  He prepares the roster in consultation with deptt. of Agriculture & horticulture etc. so that equitable distribution of releases  is made in keeping with requirement of adopted cropping pattern.

  - PROJECT PARAMETERS

    - Landmarks

Date of commissioning of UGC             8/4/1854

Date of start of canal irrigation 	         1/5/1855

Ist remodelling done in 			    1868

Osrabandi System Started                          1880

Remodelling of Capacity

Remodelling & increase in capacity

at head from 191.16 cumecs to 240.72 cumecs			1932

Increase in capacity from 240.72 cumecs to 29736 cumecs	1951-52

Construction of new headworks i.e. Bhimgoda Barrage 	1978-84

    - Modernisation of UGC System

This first time slice includes replacement of 4 major cross drainage works on main canal; lining of canal connecting the existing system from 6 KM to 36 km  & 100% modernisation in lower reaches.  A detailed map showing Upper Ganga Irrigation Modernisation Project is given at  Dwg. 1.2.

    - Command Areas

Initial gross command area		       15 lakhs acres (6.07 lakh ha)

Present gross command area		   50 lakhs acres ( 20.23lakh  ha)

Present culturable area			   25 lakhs acres ( 10.11 lakh ha)

Initial length of canal & branches in system	         1412 Km(878 miles)

Total present length	 & branches in system	                          6540 Km

    - Salient Control Points & Crossings

Head regulator for Parallel Upper Ganga Canal 	        6.04 Km

Dhanauri Regulator 				             19.288 Km of UGC

Silt ejector 							       20.14 Km

Ranipur Rao xing (Syphon) 				         8.20 Km

Pathri Rao xing (Superpassage) 				       15.65 Km

Rat mau Rao (Aqueduct) 			            20.313 - 20.562 Km

Solani River (Aqueduct) 				         28.748-29.110

Common reach of UGC &

Parallel Upper Ganga Canal        			        29.4 to 30.6 Km

    - Alignment of UGC

U.G.C. flows almost North to South in head reach till Janli where Anupshahar branch takes off; hereafter it takes South-West direction.

  - GEOMORPHOLOGY

The U.G.C. in its upper reach caters to the irrigation requirement of Muzaffarnagar district, which lies in the Doab of Ganga & Yamuna river between North Latitude 290 1.0'-50" and 290 44'20' & east longitude 770 02' to 780 7' (area 4049 sq. km)

Terrain is flat; general slope is towards south; (0.4 m / Km); highest elevation 261.0 & lowest 222.0 Meter above MSL.

This can be divided into five geomorphic units.

- Sand Bar

Along the Ganga river with occasional changes due to floods.

- Flood Plains

Flat, low lying, poorly drained features

- Ravines : Occur in western part in the form of gullies along rivers Kali Hindon & Krishni as a result of localised surface runoff eroding the unconsolidated soils.

- Young alluvial plain between western bank of Ganga & Eastern bank of  Yamuna.  Gentle slope, presence of Ox-bows, Khadir area.  The Ganga Khadir is 20 km wide in North which dwindles to 2km width in the vicinity of Bhukarhen.

- Old Alluvial Plain

- Tract between Ganga & Kali river

- Between Hindon & Kali river

- Between Hindon & Yamuna River

- Lithology and Ground Water Aquifer

Study of lithological logs reveal that rough nature of alluvial sediment is complex & there is quick alternation of fine to coarse sediments; by & large four distinct permeable layers occur down to 450 m.

3-24 m is top sandy clay bed covering almost huge area.  The first aquifer is encountered after top clay layer.  Ist & IInd aquifer  is separated by 10.15 M thick clay bands.  The IInd aquifer occurs between 115-235 m below ground level Sedimentation of this aquifer are less course.

Hydrological studies in Bijnor and Saharanpur have been carried out in the Bhabar - Tarai belt and the adjoining Gangetic alluvial plains.  In Bhabar Tarai belt ground water occurs under confined conditions. Large groundwater development is possible in Gangetic alluvium. Ground water occurs both under confined and unconfined conditions in the about lower reaches of U.G.C.