Annex. 4.1

COMPARATIVE STATEMENT OF LOSSESS BY VARIOUS

STUDIES /EMPIRICAL FORMULA /ASSUMPTION & PERFORMANCE INDICATORS

UGC

| Losses on the system as per upcip report | Losses as per predet-ermined share | Losses as per empirical formula (Roorkee Research Institute) | Losses to envisages be saved on lining of canal as per World Bank Projects | Losses as per recorded gauge /discharge at control points (%) | Losses as per field observation (%) | Delta variation on various channels | 20 km length loss on channels (% per cusecs of head Q) | Studies on Delta (1901) | Scope of improvement in efficiency if recommendation implemented for conveyance system & distribution network | Scope of increase in efficiency by change -over to modular outlets & rationalizing water allowance | Scope of increase in  by PIM & supply at bulk in Tertiary system | Scope of overall increase in  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lined System Main Canal 5% Branch Canal 10% Distributary and Minor 15% Efficiency upto outlet level 73% Overall Efficiency 51% Percolation rate in soil 1.0-5.0mm/day for soils suitable for rice cultivation Puddule/cash and 7.5 mm (unsuitable for rice) | Main/Branch 8.5 % Distributary system 7-10% No loss accounted for in tail reach of Mat branch as per revaluation by Ex. Engr., Mathura | 8 Cs/million sq. feet of wetter perimeter (2.44 cumecs per million sq. meter of wetter perimeter) | 6 Cs/million sq. feet of wetted perimeter (1.83 cumecs per million sq. meter of wetted perimeter) | Upper main canal 0.5-2.6% Main line lower 2.84-8.23% Branch 8.31-15% Disty. 10.5-16% Water Course 11.5-16.2% | Watercourse 11.5 to 16.29 (Main ) & 9 to 11 % (Branch) field chak 15-20% loss calculated water courses 4% per 100 meter length | Jatwari 2.85 Kharif 4.43 Rabi 2.17-2.25 Baldeo 4.43-6.76 1.98 - 3.23 Dagheteri 3.43-6.66 2.38 - 5.07 | Lined Loss Head Q 2.25cs - 0.22 60 cs -0.44 10.0 cs -1.18 0.6 cs -5.08 Unlined 2.25 cs -1.02 60 cs - 1.21 10 cs - 5.56 0.6 cs -24.30 | Head reach UGC Kharif-2.92 Rabi -1.50 Anupshahar Branch Kharif - 2.41 Rabi- 2.50 Meerut circle Kharif - 2.66 Rabi-1.60 Bulandshahar Divn. Kharif - 2.58 Rabi-1.83 Aligharh Division Kharif-2.60 Rabi-1.50 | 10-15% | 10%-20% | 10% | 15-35% |

Overall WUE(excluding plant transpiration

and unavoidable use-up) ; 33 to 38 %