Annex. 4.6

Some Earlier Studies & observations (1901)

On UGC

Showing variable duty in different tracts of same UGC System

Tarct					Delta

 = depth of water (feet)

Kharif			Rabi

Northern part Ganga					2.92			1.50

Anupshahr							2.41			2.50

Meerut							2.66			1.60

Bulandsharhr							2.58			1.83

Aligarh							2.60			1.50

Some earlier studies & observations

Duty of Water (Kharif 1901) - A Comparison

| Canal | Average utilisation in cusecs at the head of canal during season | B=base i.e. No. of days canal was in flow |  = Duty of discharge utilised |  = equivalent aggregate depth of water on the fields on the basis of utilised discharge (feet) | V=Volume utilised per acre irrigated (Cuft) |
| --- | --- | --- | --- | --- | --- |
| Upper Ganga Canal | 5152 | 127 | 73 | 3.41 | 150,320 |
| Lower Ganga Canal | 3133 | 141 | 67 | 4.16 | 181,830 |
| Eastern Jamuna Canal | 1310 | 169 | 120 | 2.75 | 121,680 |
| Western Jamuna Canal | 4200 | 181 | 64 | 5.58 | 244,350 |