CHAPTER - 4

ANALYTICAL STUDY OF THE LOSSES

4.1	INTRODUCTION

Daily discharge data has been compiled for a considerable period at least for the last four crop seasons; starting right from Tajewala Head Works which feeds the Western Jamuna Main Line upper.  Even though Hathnikund Barrage had been completed as per schedule in 1999; the regulation of supplies into W.J.C. were not done through new system then & even now due to delay firstly & largely due to request of State Electricity Board  through Secretary, Irrigation & Power that they were far behind in taking up of the construction works of power channel & that irrigation department may accordingly co-operate by running the old Tajewala head works.  The running of supplies from old Tajewala were thus compelled to continue even after HKB was completed. The W.J.C. link canal which too was thus delayed due to some mistake in alignment; had though been completed, the old Tajweala Headworks is being operated even today. Accordingly the transitional recurring yearly & sometimes bi-annual construction of forcing bund to feed Eastern Yamuna Canal continues as a costly and erratic feature.  Additional avoidable cost of maintenance of Tajewala headworks; appurtenant works, river protection works, upstream upto HKB etc; coupled with risk of old structure & non-utilisation despite extra O&M cost of HKB & heavy water transit losses are thus the outcome.  Loss of additional power proposed generation as such is additional feature. This discussion with Engineer-in-Chief, Haryana Irrigation Department in June 2001 & subsequent recent deliberations revealed that HKB Barrage constructed in 1999 is as yet not operative mainly due to delay by power wing of Haryana to complete its channel.  The alertness & round the clock vigil of the old Tajewala headworks by extra avoidable maintenance costs, repairs and manual regulation continues.

Losses have been worked out on the system in three stages of canal network.

  - SELECTION OF CONTROL POINTS

- D/s Cross-regulator  of old canal & Augmentation Head to Munak : - The first major & important control point for distribution of supplies into various groups of rotation.

- The groups of rotation keeping in view availability specially during lean period have been divided into,

| Through Hansi Branch | Through Parallel Delhi Branch |
| --- | --- |
| - Anta group - Butana group - Sunder group | - JLN group - Bhalaut group |

- Losses from Head Parallel Delhi Branch RD 0 to RD 145,000(0-44.2 Km) at Khubru another Cardinal regulation point from where JLN feeder, Bhalaut sub-branch & Delhi branch trifurcate as also Bhainswal Disty and Sardhana Disty. take off.

- Losses from Head Delhi Branch at RD 145000 to RD 282000 (44.2 to 86 km) & inclusive of losses from RD 0 of Delhi S/ Branch to RD 73 (22.3 km) crossing, Najafgarh Drain.

- Losses on Bhalaut S/Branch from Khubru Head to (i.e. Kanheli head) tail BSB from where Jhajjar S/Branch and Dulehra S/Branch take off.

- Losses on Dulehra S/Branch System

- Losses on a number of distributary and Minors, both lined & unlined

- Losses on water courses / field application losses are to be repeated for working out average (as also losses on lined & unlined water courses separately).

The review of analysis show that over the period of four crop seasons (Annex 4.1 - 4.8)  Kharif 99; Rabi 99-2000; Kharif 2000  & Rabi 2000 -2001 ; the daily discharge figures as released at head and that reaching various control point show enormous variations in the quantum of transit losses.  The field visits shows that transit  losses are constituted of ,

- Seepage or absorption

- Evaporation loss

- Evapotranspiration loss also where there is profusion of vegetal growth on berms; inside slopes etc.

- Leakages from head-regulators of offtakes even when they are closed or supposed  to be closed & not entitled to release

- This is occurring mainly due to defective regulating arrangement with worn-out wooden needles.

- Gates & gearing have not been installed on all the regulators even on important control points like Khubru at RD 145,000 (44.2 km) Parallel DB & Delhi Branch as also at many other important regulation points.

4.3	DETAILS OF STUDY,  BASIS AND OUTCOME

The study of losses on the W.J.C. system; right from the headworks at Tajewala to head of outlet & field application losses; have been carried out to assess water-use efficiency i.e. percentage of releases from head gainfully utilised for production of crop.

A comparative study of water released in particular regions and the area irrigated has also been done to compare water actually released with the figures theoretically needed to raise the cropping pattern. Please refer Annexure 4.11.

Study has also been carried out to compare the pattern of monthly releases or monthly percentage to the total  to ascertain excess release or shortfall to corrective measures.

Apart from the computation of percentage of loss in various stages of distribution; dissertation has been done on the behavior of canal network, headworks; headregulator; or cross regulators regime of canal; lift channels and above all the regulation & communication system.

Study has also been done to derive actual irrigation figures achieved on the ground during Rabi & Kharif on various outlets of the same channel to mark the trend of percentage intensity in head reach, middle reach and tail reach (Annex 4.15) Inspite of channel having been modernised & full supply discharge at head inequity in distribution persist, reasons being delayed maintenance, overdrawl by headreach outlets; obstructions in bed deliberate or otherwise causing heading up. Studies (results drawn at annexure 4.10, 4.11& 4.17) show even in the same overall W.J.C. command and continuos areas of similar soil characteristics & even cropping patterns; delta is different; so also are widely variant the percentage monthly releases in each crop.

Study was also conducted to evaluate how far objectives envisaged in the modernisation of W.J.C. system (Lining of canals) as well as lining of water courses under Command Area Development was achieved.

Annexure 4.12 depicts percentage increase in irrigation post  lining over irrigation figures prior to lining.  The fact that increase is not uniform as also there is decrease in some cases show that a large number of  other factors predominate & continue to interface with equity inspite of the fact that even travel time from head of disty. to its tail has considerably reduced after lining.  The travel time has reduced by as much as 30%.  See Annexure 4.14.

The variation in increase in irrigation post lining or even decrease has been attributed to following reasons.

- Some of the channels are located near head of the canal distribution system; others at tail; the fluctuation in supply; affecting head reach channels less.

- The supply during rotational turn is not uniform; on various disty system.

- The supply during various stages of growth of crop & specially during period of critical growth is variable for different channels

- Cropping pattern differ.

- Soil characteristics vary.

- Sub-soil water level conditions vary.

- Condition of channel i.e.silting, berming, vegetal growth trend to affect the hydraulics & thus performance of channel & results of irrigation.

- Incidence of unauthorized  acts and tendency of over-drawal by head reach farmers etc. differ.

- Attitudinal change, as usage by farmers & control by personnel of Department is not uniform.

- Supplementing of Irrigation by flow with ground water too affect the performance parameters.

- Quality of lining not uniform.

Considering integration of irrigation practices to get maximisation of benefits conjunctive use of surface and ground waters, motivation and switch-over to cropping pattern conducive to soil and water table characteristics; adoption of versatile freed application practices to minimise loss of   water & integration of drip & sprinkler irrigation with flow are bound to pay dividends experiments in lift areas of WJC by use of sprinkler showed 29 percent to 56 percent and saving in water over flow irrigation.

Result of irrigation by sprinkler gravity flow on Kherakheri, Disty of W.J.C. lift areas.

| Crop | Delta by sprinkler irrigation | Delta by gravity | Saving in water |
| --- | --- | --- | --- |
|  | (cm) | (cm irrigation) | (percentage) |
| Cotton | 28.60 | 40 | 29 % |
| Wheat | 14.3 | 32.5 | 56% |
| Barley | 7.70 | 17.5 | 56 % |
| Gram | 7.70 | 17.5 | 56 % |

Losses from headworks to Dadupur have been discounted.  Silt ejector is also located in the same reach & data on discharge released as such is not available. There is level crossing of Somb river with main line upper  u/s Dadupur and there is huge variation & unaccountability in figures supplied as such

Since transit loss or percentage loss would not be reliable based on such data; this reach has been discounted for the purpose.

  - ESTIMATION OF LOSSES

    - Losses on WJC main line upper

Typical Analysis of loss between Dadupur Regulator & Cross Regulater on WJC & head Augmentation Canal.

Oct 99 (On monthly water account Basis)

Releases into  Main Line Lower	7119 cumecs days

Releases into Augmentation canal 1172 cumecs days

Releases into Main W.J.C. (unlined) = 5737 cumecs days

Supply reaching bifurcation of Augmentation canal

= 5737 + 1172 =  6909 cumecs days

Loss =  7119 - 6909 = 210 cumecs days

Percentage loss = 210 / 7110 = 2.94

(On daily water account basis)

Typical 6/1/2002

Releases main line at Dadupur  = 57.8 cumecs

Supply into head Augmentation canal = 36.4 cumecs

Supply d/s cross regulator = 19.8

Loss = 57.8  - ( 36.4 + 19.8)

= 57.8 - 56.2 = 1.6

percentage loss = 1.6 / 57.8 = 2.7 %

4.4.2  Loss Between Main Line Lower, Head Augmentation Canal & Munak

Cumecs

| - Supply released in Augmentation Canal (Lined) | 2514 |
| --- | --- |
| - Supply released d/s cross regulator into WJC (unlined) | 1828 |
| - Supply drawn from old WJC (direct offtakes) | 22.5 |
| - Supply augmented from NBK link | 1449 |
| - Supply drawn by offtakes of NBK | 68.0 |
| - Supply drawn at Munak by Parallel Delhi Branch | 2247 |
| - Supply drawn at Munak by Hansi Branch | 2324 |
| - Supply drawn at Munak by Munak escape | 663 |
| (X) Supply released (A+B+D) Minus - (C+E) = for distribution at Munak | 5700.5 |
| (Y) Supplies drawn (F+G+H) | 5234 |
| Loss X - Y | 466.5 |
| Percentage loss | 8.1 % |

ABSTRACT OF LOSS BETWEEN HEAD MAIN LINE LOWER & HEAD AUGMENTATION CANAL AND LOSS BETWEEN

HEAD  AUGMENTATION AND CROSS REGULATOR TO MUNAK

| Loss Between Main line lower to head Augmentation canal |  | Bet. Augmentation canal HD/X-regulater to Munak | Remarks |
| --- | --- | --- | --- |
| Oct' 99 | 2.94% | 10.38 % | Discounted as supplies received & distributed d/s more |
| Nov' 99 | 7 % | Supply more than releases |  |
| Dec' 99 | 6 % | Supply more than releases |  |
| Jan '2000 | 4.64 % | Supply more than releases |  |
| Feb '2000 | 5.4 % | Supply more than releases |  |
| March ' 2000 | 6 % | 1.87% |  |
| April |  |  | Not Co-relative |
| May |  |  |  |
| June | 4.22 % | 8 % |  |
| July | 3.64 % | Figures of releases into Aug. canal appear wrong |  |
| August | 2.3 % | Figures of releases into Aug. canal appear wrong |  |
| Sept | 3.45 % | Figures of releases into Aug. canal appear wrong |  |
| Jan' 2001 | 6.42 % | 5.2 % |  |
| March' 2001 | 8 % | Not co-relative |  |
| April |  |  |  |
| Oct |  | 7 % |  |
| Nov |  | Discounted |  |
| Dec |  |  |  |
| Jan' 2002 |  |  |  |
| Feb |  | 6 % |  |
| March |  | 9 % |  |

INPUT DETAILS

- Discharge for various crop seasons released from Tajewala Head works shown at Annexure 4.1

- Releases between Dadupur; Augmentation canal & Munak Releases into NBK Link shown at Annex 4.2.

Releases into Hansi Branch, Parallel Delhi branch, Munak Escape, shown at 4.1.1.  offtakes of Parallel Delhi Branch shown at Annexure 4.2

- Withdrawal by offtakes of NBK; & WJC (Unlined) shown at Annex 4.3

- Withdrawal by direct offtakes of Delhi Branch shown at Annex 4.4

- Withdrawal by offtakes of Bhalaut Sub Branch, shown at Annex 4.5

- Withdrawal by offtakes of Jhajjar Sub Branch, shown at Annex 4.6

- Withdrawal by offtakes of Dulehra Sub Branch, shown at Annex 4.7

- Releases into Jua Disty & supply at tail and its offtakes, shown at Annex 4.8

- Discharge tables of various channels of WJC, shown at Annex 4.9

4.4.3	Losses on Parallel Delhi Branch (PDB) Between Head at Munak and Khubru (Tail PDB) RD O - 146000 (49 Km Length)

Study on releases at head PDB, the total of withdrawls by direct offtakes and channels taking off at tail i.e. JLN; Bhalaut, Delhi Branch, Bhaniswal disty, Sardhana Disty etc. show very huge variation; most of the time supplies being delivered as totalled up for discharges of offtakes is more than that released from head.  Taking random example from other samples for various months when the release at head is marginally or otherwise more; losses work out to between 4 to 8 %.  Exercise to depict daily losses or monthly loss has been considered futile.

Monthly data studies taken up reveal receipt for distribution exceed release at head for the month of Oct 99, Nov 99, Dec 99, Jan 2000, March 2000, June, July, Aug, September 2000 by 28.5 to 285 cumecs days.

Losses work out to 10.1 percent in March 2001, 4.5 percent in June 2001 & 12 percent in April 2000, 6.87 percent in Feb. 2000.

4.4.4	Losses between Tail Parallel Delhi Branch R.D. 145000 (44.2 km) (Khubru) i.e. Head Delhi Branch and Tail Delhi S.Branch 73000  (22.3 km) crossing Najafgarh Drain

(Distance Head Delhi Branch to Tail DB - 46 Km

Head Delhi S/Brach to Crossing Drain - 22.3 km

Total = 68.3 km

Taking into consideration the releases at head Delhi Branch as per discharge table checked on 11/9/2000 and discharge run in offtakes & direct outlets the percentage losses in the 68.3 km; work out to.

| Month | Percentage losses |  |
| --- | --- | --- |
| 10/99 | 10.8 |  |
| 11/99 | 10.1 |  |
| 12/99 | 8.6 |  |
| 1/2000 | 7.3 |  |
| 2/2000 | 10.5 |  |
| 3/2000 | 11 |  |
| 4/2000 | 11.2 |  |
| 5/2000 | 10.0 |  |
| 6/2000 | 11.9 |  |
| 7/2000 |  | Discounted |
| 8/2000 |  |  |
| 9/2000 |  |  |
| 10/2000 | Not reconciled |  |
| 11/2000 |  |  |
| 3/2000 | 11.5 |  |

Longitudinal section of Delhi Branch is enclosed as Drg. 4.1

4.4.5	Losses on Dulehra Disty System (Reach head to Tail)

Rabi 2000-2001 as per available data

|  |  | Releases at head (Average of Morning & Evening) | Offtakes plus Direct outlets | Loss (%) |
| --- | --- | --- | --- | --- |
| Losses | Oct 2K | 95 | 83.5 | 12.2 |
|  | Nov 2K | 94.2 | 83.0 | 11.9 |
|  | Dec 2K | 73.1 | 65.1 | 11.0 |
|  | Jan 2K | 43.8 | 39.3 | 10.4 |
|  | Feb 2K | 28.4 | 25.2 | 11.3 |

4.4.6	Losses on Bhalaut Sub Branch Reach head at Khubru in Tail

(RD 146000 PDB to Tail BSB)

(Km 44.2 to Tail BSB)

| Releases at Head |  | Total offtakes + Direct outlets | Loss (%) |
| --- | --- | --- | --- |
| Oct 2000 | 412 | 363 | 12 |
| Nov 2000 | 308 | 272 | 11.5 |
| Dec 2000 | 282 | 251.4 | 10.8 |
| Jan 2001 | 186.4 | - | Not reconciled |
| Feb 2001 | 95.6 | - | Water Distributed more than released |
| March 2001 | 21.1 | Data incomplete |  |

4.4.7	Losses On Distributary And Minor System Of WJC

|  |  | % loss |
| --- | --- | --- |
| Jua Disty. | Reach 0-12 | 1.5 |
| Hulambi Minor | 0-3 | 1 |
| Daboda Minor | Reach 0-6 | 1.5 |
| Lampur Disty | 0-Tail | 6.3 |
| Pai Disty | RD 42-69 | 2.5 |
| Bhainswal Disty | 0-10 | 1 |

Photographs of discharge sites are enclosed as Fig. 2 & 7.

# 4.5	LOSSES BELOW THE OUTLET IN FIELD WATER COURSE

Representative chaks of the command area of WJC fed by minors; distributaries and sub-branch were selected over a scattered area to assess losses below the outlet in the field water course.

For this purpose both lined & unlined water courses were selected.

- CHAK OF OUTLET RD 10180/R LAMPUR DISTY.

The channel takes off from Delhi Sub branch CCA of the outlet is 251 hectares acres with split up as under. Village Ghoga 206; Lampur 32 & Bankner 12.5. Annex 4.16 shows the actual Irrigation achieved & crops raised in various crops seasons.

Discharge of the outlet running at head of the water course was 1.48 cusecs (0.042 cumecs); the quantum reaching shareholder No. 116,106,107 as per warabandi in force was observed. The water course was found to be in average unlined condition; loss observed was 12.8%-16% and in the months of Dec. & May. The entire supply in water course was flowing down as per warabandi & this was in the condition when watercourse had been continuously running for a few days  & the reach length was wet. Losses in the initial running of water course; when supply released after a closure of a fortnight; were also measured; these were significantly higher for a very short period; but stabilised; but since the fields made observation for the purpose were also nearer to source; the reach of measurement was merely 150 meters and loss in the initial running; of as high as 8-9 and stabilized to 4%.  Intensity of irrigation in Kharif 99; Rabi 99-2000; Kharif 2000 & Rabi 2000-2001 as actually achieved has been 58.3%; 59.1%, 58.1andand 57.0 and respectively. Major crop during Kharif being Rice & vegetables and in Rabi; Wheat /Sarson.

Total cusec days run on the Lampur distributary during Rabi 99-2000; Kharif 2000 & Rabi 2000-2001 have been 2.56; 5.20 and 2.65 cumecs days respectively; whereas area irrigated in hectare during these crops respectively has been 148, 146 & 143 hectare. Average Delta of multi crops consisting of Jawar, Rice, Vegetables & garden flowers during Kharif and wheat, sarson; Barseem etc. in Rabi works out to 0.487'; 1.14'; 0.52' i.e. almost double in Kharif.

Outlet chak 14660 R Jaunti Minor (0.043 cumecs)

Authorised discharge 1.49 Cusecs

CCA (234 hectare)

System PA1 Disty taking off at RD 228 (69.5 km) of Delhi Branch

Losses observed on the water course were 14.8 and & 20.5 and during December & June for the watercourse of chak for above outlet. Warabandi is being implemented  satisfactorily.

The delta respectively for Rabi 99-2000 Kharif 2000 & Rabi 2000-2001 works out to 0.18, 0.28; 0.26. The variation in supplies received for Irrigation on Lampur disty & Jaunti Minor  would clearly show the disparity and obvious reasons. Lampur Disty draws supply directly from Delhi S. Branch whereas Jaunti Minor is at the tail end of Pai Disty; it off takes at RD 69310 of Pai Disty; which takes off from Delhi Branch RD. 228 i.e. the travel of Lampur outlet RD 10880 is mere 3.3 KM from S/Branch where as that of outlet 14660 Jaunti Minor  is 28 km. from the system offtaking from Delhi Branch. Delta variation for Rabi 99-2000 & Rabi 2000 –2001 is due to total days run from Oct. 99 to March 2000 being 29 against 46 days in the period from Oct 2000 to March 2001.

Losses on Lined & Unlined Watercourses

Lining of water courses in the WJC command was taken up under CAD programme. On the basis of experiments carried out in the state to determine optimum length to derive maximum saving in transit loss consistent with  reasonable cost it was determined that optimum length works out to 10,000 (3000 metres) feet beyond which the efficiency or savings were not proportionate to higher additional cost.

The result of lining of water course in Gurgaon canal command of WJC as well as Rewari lift irrigation area where soils are sandy loam showed a (1.96 to 2.61 cumecs) marked saving in the transit loss. This corresponded to 6 to 8 cusecs against 3.26 to 3.92 cumecs per million sq.m of  wetted perimeter) after lining but soon after even after two years of lining & lapse of two monsoons vegetal growth in joints; vegetation on slopes & lack of good upkeep has brought down the efficiency & losses have gone up.

Losses on an average on water course lined but after 13-14 years of lining as observed now on Ferozepur Jhirka Disty. Sikri Disty, Kheri Disty; Nuh Disty showed losses remain substantial & from outlet head to last chak of shareholders; these range between 10.5 to 16 and on unlined water course in the command of WJC on Hulambi Minor, Sultanpur Minor; losses were of the order of 12 to 16%. Percent losses under different Irrigation condition is given at Table 4.1

Chak plan of outlet 10180 lampur; 274 Delhi S/Branch and Jaunti Minor are enclosed as Drg.  4.2, 4.3 & 4.4.

Pre lining & Post lining Irrigation figures depicted on Annexure   4.12

Seepage loss	                     			        Annexure   4.13

Efficiency in saving in Time of water

delivery after lining.					        Annexure  4.14

4.6	LOSSES ON FIELD

TABLE - 4.1

Percent Losses under different Irrigation Condition

(Haryana Agricultural University (HAU) Studies Publication 1983)

| System Characteristics | Canals | Distributaries | Field Water-courses | Field Loss | Total Loss | Net Utilisation |
| --- | --- | --- | --- | --- | --- | --- |
| - Entire System Unlined | 15 | 7 | 22 | 27 | 71 | 29 |
| - Only canals lined | 4 | 7 | 25 | 30 | 66 | 34 |
| - Canals & Disty. Lined | 4 | 2 | 26 | 32 | 64 | 36 |
| - Whole System Lined | 4 | 2 | 6 | 42 | 54 | 46 |
| - Whole system lined & Sprinkler irrigation | 4 | 2 | 6 | 6 | 18 | 82 |

LOSSES IN THE FIELD

Apart from transit losses in the conveyance system & the distribution network accompanied with avoidable leakages & losses due to structural inadequacies; regulation failure or non-automation etc.; studies conducted on the overall deliveries of canal water released from the headworks & that deliverd below the outlet to the turn-out of field through a water course indicate that major & maximum component of loss generally occurs in the field itself.

Studies had been conducted by HAU for areas under Western Jamuna Canal as well as Bhakra command & it was observed that very high losses occur in the field as well as unlined watercourse.  Table 4.1 refers the present study to establish high losses in  the field below the water course.

Prevailing Conditions

Water is supplied as per existing water allowance of  0.18 cumecs/1000 hectare of CCA on W.J.C. and the Warabandi in operation entitles a farmer to receive a quantum of  canal supplies  proportionate to his area irrespective of the type of crop grown; the type of soil characteristics; ground water condition; slope of soil; climate , method of application and (generally check method is resorted to); though boulder strip is also applied.

# Water Applied to The Plant Proper & its Consumptive Use

The Water Use Efficiency has been worked out on the conveyance and distribution system right upto delivery in the command Chak of the outlet under cultivation & application by different methods.

Net Water Use Efficiency is the efficiency after application of water in the root zone of crop & avoidable wastage by evaporation over crop canopy & in filtration below the root zone. These have to be assessed to work out actual gainfull utilisation of water released.

In-filtration & Redistribution

Infiltration of Irrigation in the soil profile is modelled by employing empirically and physically based concepts. In the empirical approach; the infiltration process is not given any time distribution and soil profile is treated as a tipping bucket. Each layer is assumed to fill to its field capacity wetness instantaneously and remaining water is cascaded to lower layers. Any water passing beyond the bottom layer is lost to deep drainage.

As with in-filtration, the movement of water in rootzone has been estimated in a simplified way by transferring water from one layer to another. Transpiration (T) is the most important component of water balance from standpoint  of  bio-mass production. The first step in simulating T is the estimation of evaporative demand Eo which is satisfied by interception and soil evaporation and transpiration. Estimation of Eo  with Penman (48); Priestley & Tayler (72); and Monteiths (65) method makes use of daily meteorology variables and the diffusive resistance of canopy. Because of the assumptions & approximation, daily pan evaporative values have been used for computing Eo.

The Pan-evaporative value are available for WJC Command in Haryana & UGC in (UP). These; for various crops are depicted at Annexure 4.14.1

With these values taken in the command which are quite representative keeping in view agro-climatic; cropping pattern, irrigation scheduling prevalence and growing stages of crop and with; delta available & annexed for both the system it has been possible to work out percentage gainful utilisation in the root zone: remaining having been considered to be lost in deep drainage by infiltration & avoidable extra evaporation.

The statistics of delta worked out on Western Yamuna Canal as appended with the report shows its variation for Kharif between 1.43 to 2.92 feet & for Rabi between 0.66 to 2.04 feet for the channel system on WJC fed Gurgaon Canal; Hansi Branch etc.  The effective moisture storage varies from 45 mm to 128 mm whereas effective rainfall is limited to 300 mm.  Taking ET0 for the zone Rabi 604 mm, Kharif 1165 mm for the type of popular crop & evaporation from the root zone soil; the water actually utilized for maturity of crop is vastly variant.  Rational data has thus been taken both for the delta; ET0 etc. and average avoidable loss keeping in view of various crop values, has been taken as 25% & overall Water Use Efficiency works out to very between 36 to 43%.

Recommendations for Improving Water Use Efficiency by minimising loss below the watercourse on field itself

Land Levelling

Proper grading of land to be irrigated is essential for efficient application of irrigation water ; disposal of excess rain water in humid & sub-humid areas conserving rain water in arid and semi-arid areas.  Unevenness & undulating terrain & fields contribute to low efficiency. However, criteria & extent of land levelling for water use efficiency is guided by soil profile, land slopes; source of irrigation water; method of irrigation, climatic condition & type of crop to be grown.

Studies conducted in Haryana Agriculture University  in plots of 35 x 7.5 m size by establishing five different levelling indices i.e. 1.2, 2.0, 2.5, 3.0 3.7 and under each levelling index; two irrigation treatments i.e. applying only a fixed 6 cm depth & applying enough water to irrigate the plot adequately such as none of it was left unirrigated. Table 4.2,4.3 and 4.4  show results.  The application and distribution efficiencies are much higher with 6 cm fixed depth of irrigation water applied than when plots are completely covered (Table 4.2) However, storage efficiency is lower in fixed depth treatment.  In both the treatments however, three efficiencies decreased as the levelling index increased.  The application & distribution efficiency in the best levelled treatment  1.2 cm were 15 to  20 and higher as compared to 3.7 cm levelling index.

Grain yield & water use efficiency versus levelling index

The three-year average data of Wheat and Bajra grain yield & water use efficiency showed that yields reduced considerably under both the irrigation treatments as the levelling index increased.  However if irrigation water is not a constraint & farmer tends to overuse it (thus able to waste it); then by applying complete irrigation or excess water; loss of yield due to poor levelling can be minimized. The amount of total water used in Wheat crop showed that about 50 and more irrigation water had been applied in 3.7 cm levelling index under complete irrigation treatment  as compared to only 30.6 cm in 1.2 cm levelling index and fixed 6 cm irrigation treatment (Table 4.3). It was observed that considering yield; cost of levelling, amount of irrigation water applied &  water use efficiency/  the optimum value of levelling index should be 2.5 cm where irrigation water is limited.

Table - 4.2

Irrigation Application, Storage and Distribution

Efficiency under different levelling Index

| Levelling Index(cm) | Application Efficiencies (%) | Storage Efficiencies (%) | Distribution Efficiency (%) |
| --- | --- | --- | --- |
| 6 cm depth Irrigation Treatment |  |  |  |
| 1.2* | 78.5 | 62.0 | 93.3 |
| 2.0 | 73.0 | 71.2 | 90.7 |
| 2.5 | 69.8 | 65.9 | 86.2 |
| 3.0 | 67.0 | 64.4 | 82.5 |
| 3.7 | 65.7 | 62.8 | 72.3 |
| Complete or adequate Irrigation Treatment |  |  |  |
| 2.0 | 67.6 | 68.0 | 88.7 |
| 2.5 | 63.6 | 67.5 | 80.6 |
| 3.0 | 60.0 | 68.1 | 77.3 |
| 3.7 | 52.6 | 72.2 | 67.9 |

* In 1.2 cm (levelling Index), the actual depth of Irrigation was 5.2 cm.

Wheat & Bajra grain yields (q/ha) and Water Use Efficiency (WUE) (Kg/ha-cm) in different levelling index under 6 cm and complete irrigation (3 years average)

TABLE - 4.3

| Levelling Index | Yield & WUE in 6 cm & complete Irrigation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Wheat |  |  |  | Bajra |  |  |  |
|  | Yield |  | WUE |  | Yield |  | WUE |  |
|  | 6 cm | Complete Irrigation | 6 cm | Complete Irrigation | 6 cm | Complete Irrigation | 6 cm | Complete Irrigation |
| 1.2 | 46.6 | - | 166 | - | 37.3 | - | 120 | - |
| 2.0 | 42.2 | 44.2 | 138 | 136 | 34.2 | 36.6 | 106 | 117 |
| 2.5 | 39.3 | 41.9 | 128 | 120 | 32.9 | 34.1 | 99 | 99 |
| 3.0 | 36.5 | 40.2 | 116 | 108 | 31.4 | 29.6 | 92 | 82 |
| 3.7 | 34.9 | 37.3 | 110 | 92 | 27.9 | 29.7 | 84 | 83 |

Table - 4.4

Total Water Use (cm) in different levelling Indices

under 6 cm & complete irrigation

| Levelling Index | Water Use (CM)* |  |
| --- | --- | --- |
|  | 6 CM | Complete Irrigation |
| 1.2 | 30.6 | - |
| 2.0 | 34.0 | 38.3 |
| 2.5 | 34.4 | 40.8 |
| 3.0 | 34.9 | 42.8 |
| 3.7 | 35.4 | 47.0 |

* Total water use is inclusive of irrigation water, rainfall & profile water

    - Field Loss Observation on Selected Chaks of W.J.C.

After canal water was released in the Naka of the individual field; loss depends on the type of preparation of field; soil characteristics; type of crop; slope; weather conditions; stage of watering; sub-soil water level etc. Study on field loss i.e. water delivered to the field & needed for evapotranspiration or consumptive use to raise the crop would determine efficiency.  The parameters are complex so also conditions from place to place.  The Delta figures actually worked out & appended show these figures vary during Kharif and Rabi as also for the same crop from channel to channel as discussed in field watercourses loss on Lampur Disty. and                      Jaunti minor comparing the volume of water released during turn of  a shareholder as per his warabandi on Lampur. Disty. and Jaunti this too again based on releases as per discharge run in the disty and minor during the crop season; converting it into depth of water and the average of delta actually worked out on  the channel for cropping pattern shows lot of variation.  Such estimation has also been done for Jaunti Minor and a number of other channels.  It is difficult  to assign percentage for wasteful use when water was run but not needed & study being carried out only during day time the minimum of loss on the field application can be taken as 18 and to 22%.  This however; loses significance in view of observation as below; as discharge of outlets very from chak to chak.

Another study in a field chak on a freshly ploughed up area was taken up for the field measuring 50 x 50 approximately. 0.6 cusec of Canal supply was released. Time taken to water the field was observed then discharge increased. The measured discharge was 1.05 cusecs.  It was noticed that 0.6 cs release of water requires, 1.40 times the volume of water to cover the same area as when discharge of 1.05 cs was released.  This experiment was conducted on sandy. loamy soil; on loamy soil this figure would be different.  A similar study on comparatively sandy soil resulted in manifold volume of water with 0.5 to 0.6 cusecs than with a discharge of  about 1.0 cs.

  - ABSTRACT OF AVERAGE OF TRANSIT LOSS (percentage)

(Based on Gauge/Discharge maintained at Control Points of W.J.C. System)

| S.No. | Losses in Main Line Upper bet Dadupur & head Augmentative canal | Losses between Head Augmentation canal (including release in unlined W.J.C. below cross regulator ) and Munak i.e. RDO of Parallel Delhi Branch (PDB) | Losses between Munak i.e. R.D.O. of PDB & R.D. 145,000 (i.e. offteake of JLN, BSB,DB) |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| 1 | 4.6 | 8.5 | 8.3 |

| Losses between RD 145,000 at Khubru to RD 73 of Delhi S. Branch (i.e. RD 145-282 DB and 0-73 DSB) | Losses between Head & Tail Bhalaot Sub-Branch | Losses between Head & Tail Dulehra Sub-Branch |
| --- | --- | --- |
| 5 | 6 | 7 |
| 10.4 | 11.4 | 10.2 |

| Losses on Distributory / Mr. System | Loss on unlined field water course | Loss on field water course lined 10 years or more back |
| --- | --- | --- |
| 8 | 9 | 10 |
| 12 | 16 | 13 |

| Loss on lined water course | Farm / Field Application loss | Average of column (4+5) | Average of coloumn (6,7,8) |
| --- | --- | --- | --- |
| 11 | 12 | 13 | 14 |
| 7 | 20 | 9.35 | 11.2 |

Water use efficiency (with lined distribution system; unlined water course) =48%

Water use efficiency (with lined water course) = 51.7%

Accordingly recommendations are being made to adopt field application methods under different conditions to optimize water-use efficiency.  Water use efficiency can also be increased by improving upon crop and soil management practices.

Since maxm losses occur below the outlet and on field or farm itself two chapters have been added to adopt field application practices in accordance with area-specific conditions and improving upon crop and soil management practices.

4.8	RECOMMENDED METHODS FOR FIELD APPLICATION UNDER DIFFERENT CONDITION FOR OPTIMISING WATER USE EFFICIENCIES

Improper methods of  water application does not result merely in water loss due to percolation beyond the reach of crop root zone but also a loss of nutrients due to leaching.  This also results in non-uniform distribution & reduced crop yields.  Efficient water application is one that enables uniform application of water according to the need of the crop with its minimum loss and at rate and frequency consistent with intake water retention characteristics of the soil.  Important factors for choice of method are : -

| i) Crop Species | Rooting depth; consumptive use and peak daily water use; critical stages for watering criteria |
| --- | --- |
| ii) Climate | Temperature; relative humidity wind velocity; rainfall |
| iii) Land topography | Depth of soil; land preparation cost |
| iv) Soil Characteristics | Water intake & retention properties |
| v) Irrigation Water Supply | Canal supply : - Adequacy of water, size of stream, Delivery schedules, water quality |
| vi) Any problem | Salinity / alkalinity' high Sub-soil water level; Drainage |

Irrigation Efficiency:  Entire water applied for irrigation can never be stored in the root zone; even in the most versatile of application techniques; losses do occur due to non-uniform distribution  over the field;  waste at the end of borders and furrows ; some percolation below the root zone depth & in case of sprinkler irrigation; evaporation from the spray and retention on the foliage.

For selecting and evaluating performance of any Irrigation method; the three irrigation efficiencies i.e. application; storage & distribution have been kept in view.

Border Irrigation

Field experiments in sandy loam soils were conducted to select optimum length, width & slopes of border strips.  It was observed that grain yield & WUE of wheat was 8.3 & 8.4 and respectively higher in 70 M long borders as compared to 50 M but  with regard to slope, the level border gave higher yields than steeper slopes.  The WUE decreased with increasing width & decreasing slope (Refer Table 4.5).  In respect of width of borders, higher wheat yield was obtained in 8 M wide borders than in 10 to 12 M wide borders.  Thus where water is scarce & considering WUE surface drainage needs in Kharif season & also keeping in view the land grading cost,  border strips of 70 M long; 7.5 M width & 0.1 and slope were found to be most suitable.  However, where water availability is not a constraint level borders of same width & length result in better performance.

Border method is more suitable to soils having moderately low to moderately high infiltration rate; not used in coarse sandy soils with high infiltration rates because of stringent limitation in design; also not suited to soils with low infiltration rate; since to provide adequate infiltration opportunity time without surface runoff at the lowest end; the irrigation stream may be too small to completely cover the border strip.

The border strip is suitable to irrigate all close-growing crops like Wheat; Barley, Fodder crop and legumes however, not suitable for crops like rice which requires standing water.

Table - 4.5

Wheat yield (q/ha) & water use efficiency (q/ha-cm) in border strip with different slopes & length.

| Slope | Yield |  | WUE |  |
| --- | --- | --- | --- | --- |
|  | 70 m | 50 m | 70 m | 50 m |
| 0.2 | 30.5 | 30.3 | 1.28 | 1.11 |
| 0.1 | 32.3 | 28.3 | 1.07 | 0.93 |
| Level (0) | 34.2 | 30.2 | 1.00 | 0.86 |
| Mean | 32.3 | 29.6 | 1.08 | 0.97 |

Table - 4.6

Yield of Wheat (q/ha) in border strips of different border widths

Border Width (m)

| Years | 8 | 10 | 12 |
| --- | --- | --- | --- |
| I | 40.3 | 35.5 | 32.8 |
| II | 37.8 | 31.0 | 33.1 |
| Mean | 39.0 | 33.2 | 32.9 |
| Irrigation Water Application (cm) | 23.0 | 23.6 | 24.4 |

TABLE - 4.7

Recommended slopes, stream size & width of

border strip for different soils

| Soil Type | Infiltration (cm/hr) | Slope (%) | Width (m) | Discharge (Litre/Sec) | Irrigation depth (cm) | Irrigation Time (Minutes) |
| --- | --- | --- | --- | --- | --- | --- |
| Heavy | < 0.5 | 0.05 | 7-8 | 10-12 | 7 | 50-55 |
| Medium | 0.5-1.0 | 0.05-0.10 | 6-7 | 12-15 | 6 | 30-35 |
| Light | > 1.0 | 0.2-0.5 | 5-6 | 15-20 | 5 | 15-20 |

- Border strip is more suitable for medium textured soils.

- Length may be 60-70 m.

- Inflow is to be stopped when water has advanced to 80andlength of the border.

- When available discharge is much more than required two or more strips may be watered simultaneously.

- When land slope is more than recommended & obtaining design slope is difficult either reduce the discharge or increase the width.

Check Method of Application

Water is applied individually to small & relatively level plots sorrounded by ridges so that required depth is impounded rapidly in the field.  In rolling topography, plots follow contour & application called contour checks. Deep percolation is directly proportional to the water spread in the plot.  Thus largest available non-erosive stream be adopted for irrigation so that spread time is shortest.  In this method deep percolation losses can be estimated by  an equation & also with help of monograph.

Rough rule being that water spread in the entire plot should be covered in one fourth of the time required so as to infiltrate the net depth of irrigation.

Table - 4.8

Recommended Check Sizes for different soils & size of stream

Soil Type			Stream size (litres/ Sec)

14.0                         28.0

Heavy				200-240		350-400

Medium			100-125		150-180

Light				16-20			20-25

Plot size may be rect / square but ratio of length / width not more than 4:1.

Furrow Irrigation

This is used to irrigate row crops with furrows developed between the rows in the planting and cultivation  process.  Water in the furrow contacts only 1/2 to 1/5 of land surface thus reducing puddling and crusting of soils &  renders early  cultivation. It is more suitable to crops sensitive to ponded water.  When salinity is a problem; double row beds alternate furrows avoiding salt accumulation near the young  seedling.  Furrows are most commonly made down the slope but when land slope exceed the safe limit of soil erosion they are made on contour.  These are effective in area needing surface drainage.  These also are effective when rain water is to be conserved.  In scarcity areas of water; system of alternate/skip furrow  burrow irrigation results in considerable saving of water.

Furrow stream : - Maximum non-erosive stream size is estimated by

Qm = 0.6 litres / sec

Qm = maxm non-erosive stream size in litres / sec

S    = furrow slope

Sprinkler Irrigation

- It is suitable for almost all crops except rice & Jute.  One or more of following make it a better choice than surface irrigation

- Sandy soils or soils with very high infiltration rate (more than 7.5 cm / hr)

- Shallow soils; the topography of  which prevents desired levelling

- Small stream & scarce water

- Undulating topography, sand dunes, too steep to level

- Labour scarce & costly

In Western Yamuna Canal command areas in districts of Bhiwani, Mohindergarh, parts of Rohtak, Jhajjar, Hissar : Sprinkler Irrigation would be appropriate.  This would add to savings in Water.

4.8.1	Increasing Water Use Efficiency by Improving Upon Crop & Soil Management Practices

Water Use Efficiency (WUE) Crop yield/Evapotranspiration of crop area is influenced by crop and soil management practices.  The numerator i.e. yield can be changed by management practice whereas denominator is more difficult to control.  WUE is not closely dependent on the water available if the supply is within the evapotranspiration limit; even though the crop yields depend on the adequacy of water supply.  Storage of more water in the soil profile increase greatly the WUE of grain crops grown under conditions of limited water.  Allowing an irrigated grain crop like wheat or maize run out of water at the critical phases of its growing may reduce yield & WUE drastically without lowering appreciably the total seasonal evapotranspiration.

Selection of species

One of the basic approach of increasing crop yield & WUE in a particular growing environment can be selection of plant species adopted to the total amount and distribution pattern of water.  Plants vary greatly in requirement of water.

Productivity of  Cereal per unit of water (An experiment)

| Crop Species | Water Requirement cm | Yield (Kg/ha) | Productivity of water kg/ha/cm |
| --- | --- | --- | --- |
| Rice | 120 | 4500 | 3.7 |
| Pearlmillet (Bajra) | 50 | 4000 | 8.0 |
| Maize | 62.5 | 5000 | 8.0 |
| Wheat | 40.0 | 5000 | 12.5 |

The plant characteristics which influence water requirement are leaf area; extent of root system; length of growing season; number, distribution and size of stomata, inclination of leaves specis ; selection, Plant choice & breeding can enhance the WUE.  Predictions on moisture availability, rainfall, ground water aid improving WUE.

Planting pattern i.e. density and row spacing; enabling the plant canopy more effective in intercepting radiant energy and shading weeds; reduction in rain drop impact on soil structures aid the process.  Dwarf variety of row crops benefit more from narrow rows than tall late maturing varieties.

Planting dates are extremely important cultural practices in efficient use of irrigation water.  Criteria being to choose date of sowing to ensure good germination by placing seed in the optimum moisture zone.  This is so very true & essential in semi-arid / arid zones of Western Jamuna Canal .

WEED ELIMINATION

Weeds compete with crops for soil nutrients, water as well as light.  Water requirement of weeds is greater than that of crop.  Competition begins when root system of weed & crop overlap. Eradication & Control of weed is necessary to increase WUE of crop.

Efficient crop & Water management to increase WUE includes judicious use of Chemical pesticides in producing healthy crop plant.

Tillage - Principal effect of tillage are the preparation of seed bed conducive to the germination of seed and growth of seedling; conservation of soil moisture & providing adequate soil depth for optimum root growth.

Response of fertilizer to Irrigation : Soil moisture and nutrient availability have a close relationship.  Maximum benefit from fertilizer application can be obtained under Irrigated agriculture. With adequate nutrient supply; plants which are limited in growth due to moisture stress would have a higher content of mineral nutrients than plant under comparable fertility but  not limited in growth by moisture supply.

A detailed module of fertilizer application; after evaluating nutrient deficiency; calibration of dose; interval; control & variation should be available to the training & visit Staff to educate the farmer in enhancing overall WUE.

Categorization of Command Area for Irrigation

For assessment of its capacity to withstand intensive Irrigation  without deterioration ; its continued productivity per unit of water; soils are grouped on physical, chemical & climatological groups such as,

Physical

i) Texture ii) Permeability iii) Non-capillary porosity iv) Water table v) Soil depth vi) Topography vii) Water holding capacity

Chemical

i)  Soluble salt content ii) Exchangeable Sodium iii) PH

Climatological : i) Aridity ii) Cropability

Loamy soil texture is ideal for irrigation; clayey hinders leaching & results in poor aeration while sandy results in waterloss.  High values of water holding capacity, depth of soil & non-capillary porosity are desirable characteristics for irrigation as well as drainage.  The deeper the water table; better the land as waterlogging, salinity & alkali hazards are avoided.

However, there is hardly any choice over the existing project area already catered in the command.  But above criteria & considerations help in extending irrigation as well as reclamation of degraded lands.

4.9	DELTA STATISTICS ON VARIOUS SUB-SYSTEM OF W.J.C. AND W.J.C. AS A WHOLE

The statistics of actual amount of volume of water delivered during a crop season in feet or meter per unit of area irrigated has been depicted in Annex 4.10.  The variable figures during Kharif and Rabi shows large variation in supply due to more availability in monsoon & different delta reflects variable availability on different sub-system of  W.J.C. as well as variable soil character etc.  Percentage intensity of  irrigation on W.J.C. system & actual irrigation achieved vis-à-vis other systems has also been shown in Annexure 4.17 from 1966 to 1949 for group of years.  Delta statistics indirectly shows account of the losses; when compared to ideal data theoretically required as indicated in the report.  The delta also reflects wastage or avoidable loss of water or excessive infiltration rate given the parameters of soil characteristics, subsoil water level, type of crop sown, slope of land, rainfall incidence & weather condition; the reason for excessive delta can be pinpointed.  Delta has been depicted for W.J.C. as a whole, for Gurgaon canal command, for lift area command for Hansi Branch, for Hisar major Disty. System, for Hulambi Minor; Lampur Mr. Sardhana Mr,. Turkpur Mr. Sonepat Disty; Kakroi Disty of Delhi Branch system.

DELTA OF CROPS

Value of  (Delta includes evap. & percolation loss)

Sugar Cane				120 cm (48")

Rice					120 cm  (48")

Tobacco				75 cm (30")

Cotton					50 cm (20")

Vegetables				45 cm (18")

Wheat					30 cm (12")

Maize					25 cm (10")

Fodder				22.5 cm (9")

Delta Statement on various sub-system of WJC & overall shown in Annexure 4.10 & 4.11.

    - Observation for Improving Periodic Deliveries

Pattern of releases on a sub-system has been described to eschew wastage of water when not needed & to release adequate supply at time of need for qualitative production.

percentage of increase in Irrigation achieved as much as 18.52 on a sub-system of W.J.C. by Water Management drive in 1996-1997 shows scope of Improvement.  These projections are attached at Annex 4.17

Pattern of Releases on a WJC Sub System

Existing availability of Canal Water in Command of Sewani Canal

(Typical Study)

| Region | Gross Irrigated area (ha) | Canal Water Released during year (ha cm) | Hectares Cms of Canal Water / ha of Irrigated Area |
| --- | --- | --- | --- |
| I | 5128 (29.50) | 149913 (35.4) | 29.0 |
| II | 2411 (13.90) | 518680 (12.3) | 22.0 |
| III | 3775 (21.70) | 86859 (20.3) | 23.0 |
| IV | 3166 (18.25) | 71099 (16.8) | 22.0 |
| V | 2890 (16.65) | 64485 (15.2) | 22.0 |
| Total | 17390 (100) | 424221 (100) | 24.0 |

Figure in parenthesis show percentage of the gross Irrigated Area & Water released.

Monthly existing Canal Water Supplies in the Sewani Canal System (Haryana) (Ha. Cm.)

| Month | I | II | II | IV | V | Total | Percent |
| --- | --- | --- | --- | --- | --- | --- | --- |
| May | 6255 | 1357 | 1347 | 1187 | 1799 | 11945 | 2.8 |
| June | 8021 | 660 | 650 | 650 | 2998 | 12979 | 3.0 |
| July | 11120 | 523 | 9246 | 7146 | 11720 | 39755 | 9.4 |
| Aug. | 1755 | 2861 | 2257 | 1957 | 5894 | 14724 | 3.5 |
| Sept. | 20940 | 5076 | 15551 | 11551 | 8028 | 61146 | 14.4 |
| Oct. | 20104 | 6086 | 16969 | 12969 | 8422 | 64550 | 15.2 |
| Nov. | 11088 | 5648 | 3181 | 4081 | 3504 | 27502 | 6.5 |
| Dec. | 20829 | 6148 | 9800 | 8800 | 3001 | 48578 | 11.5 |
| Jan. | 11675 | 2557 | 6206 | 5806 | 6032 | 32276 | 7.6 |
| Feb. | 14071 | 13154 | 2576 | 2076 | 650 | 32527 | 7.7 |
| March | 16051 | 5700 | 14506 | 11506 | 11375 | 59138 | 13.9 |
| April | 8004 | 2095 | 4570 | 3370 | 1062 | 19101 | 4.5 |
| Total | 149913 | 51865 | 86859 | 71099 | 64485 | 424221 | 100.0 |
| % Age | 35.4 | 12.3 | 20.3 | 16.8 | 15.2 | 100.0 |  |

The pattern of periodic and regional supplies shown in above table : reveals that quantity of water released during Kharif was 37.6 of total supply & was maxm in the month of Sept.  The quantity of water released during Rabi was 62.4 and being in the month of  October.  This shows delivery was not in accordance with water requirement by crops & water distribution was highly uneven.

For example during April to June only 10.3 of total annual water supply was released & was low in the month of May when cotton is sown.  Again regional variation (Region I to V) was also high.

Annexure 4.17 shows percentage increase of irrigation over the preceding year by special water-management drive in 1996-97.  Increase is as much as plus 18.52%.